Onsurface:

it sounds like you are looking for the exact Surface Mount Device (SMD) components to use for your ESP32 board!

When you are designing a PCB that you eventually want to manufacture (or hand-solder), you can't just pick a generic "resistor" or "chip." You need the exact Manufacturer Part Number (MPN).

Here is the ultimate cheat sheet for the standard, highly available SMD components for the ESP32 board we discussed. You can copy and paste these exact MPNs directly into the Flux.ai part search to drop them into your schematic, or use them to buy the physical parts from sites like Digi-Key, Mouser, or LCSC.

1. The Core Brain & USB

- The Wi-Fi Module:

  - MPN: ESP32-WROOM-32E-N4 (Espressif Systems)

  - Why this one: The "E" version is the newest, most stable revision. "N4" means it has 4MB of flash memory, which is standard.

- The USB-to-UART Bridge (For Programming):

  - MPN: CH340C (WCH)

  - Why this one: The CH340C specifically has a built-in crystal oscillator. This saves you from having to design a messy external clock circuit. It's incredibly cheap and reliable.

- The USB-C Connector:

  - MPN: TYPE-C-31-M-12 (Korean Hroparts) or USB4105-GF-A (GCT)

  - Why this one: These are standard 16-pin SMD USB-C receptacles. They are much easier to hand-solder than the full 24-pin versions, while still giving you the pins needed for USB 2.0 data and power.

2. Power Management (5V to 3.3V)

Because the ESP32 can pull a massive amount of current (up to 500mA) when transmitting Wi-Fi, your 3.3V regulator needs to be beefy.

- The 3.3V Voltage Regulator (LDO):

  - MPN: AP2112K-3.3TRG1 (Diodes Incorporated)

  - Why this one: It is a massive upgrade over the older AMS1117. It has a very low dropout voltage and easily handles the ESP32's power spikes without overheating. It comes in a tiny SOT-23-5 SMD package.

- The Protection Diode (Schottky):

  - MPN: B5819W or 1N5819HW

  - Why this one: Comes in an SOD-123 SMD package. You put this between your USB 5V and your circuit to prevent power from flowing backward and frying your laptop.

3. The Passives (Resistors & Capacitors)

For resistors and capacitors, you don't usually need a specific MPN. Instead, you search Flux.ai (or your supplier) by Value and Package Size.

If you are hand-soldering this board, use the 0805 or 0603 package sizes. Anything smaller (like 0402) looks like a speck of dust and is a nightmare to solder without a microscope.

- USB-C Pull-Down Resistors: 5.1kΩ, 1%, 0603 package (You need two of these for the CC1 and CC2 pins).

- Pull-up Resistors (I2C, EN pin): 10kΩ, 1%, 0603 package.

- Bulk Decoupling Capacitor (For the Regulator): 10µF, 10V, Ceramic, 0805 package.

- Noise Filtering Capacitors (For ESP32 VCC): 100nF (0.1µF), 25V, Ceramic, 0603 package.

4. The Buttons (EN and BOOT)

- SMD Tactile Switches:

  - MPN: TS-1187A-B-A-B or search for 3x4x2.5mm SMD Tactile Switch.

  - Why this one: These are the standard, ultra-low-profile rectangular buttons you see on almost all modern developer boards (like Adafruit or Sparkfun boards).

Pro-Tip for Flux.ai: When you search for these in the Flux library, look for the little badge that says "Has Footprint" and "Has 3D Model". This guarantees that when you switch from your schematic to the PCB layout, the physical copper pads will actually be there for you to route!